SHED NO 27, SHUBHAM INDUSTRIAL ESTATE, NR. TORENT POWER SUB STATION, KATHWADA GIDC, AHMEDABAD – 382430 GST NO: - 24ARDPD2565K1ZX

info@torqpackagingsolution.com

www.torqpackagingsolution.com

DATE: - 09/12/23

To,

MR: JAI KISHAN SIR.

SUB: - AUTOMATIC 30 BPM BOTTELING PROJECT

Dear Sir,

Many thanks for your valuable inquiry of above subject. TORQ PACKAGING  Solutions  Is engage in making preliminary Bottle Packaging machines like bottle filling , capping , labeling, Automation, etc… in Ahmedabad, We are having dedicated team for manufacturing, customized machine development, Sales, servicing and back office.

Thanks & Regards

Ronak Dave

+91 80000 32128

TECHNICAL SPECIFICATION OF R. F. C. MACHINE 
(Capacity: 30 BPM)

Rinsing Head 					: 6

Filling Head 						: 6

Capping Head 					: 3

Capacity 						: Min 30 BPM @ 1000 ml

Suitable Cap Shape 				: Plastic Cap

Bottle spec 						: 200 ml – 2 Liter

Air Conveyor 					: 12 Feet

Electricity 						: 5.0 KW

Dimensions (W x L x H) 				: 6 Feet X 7 Feet x 6 Feet

Weight 						: 800 Kg Approx.

Bottle Dimension 					: 95 mm (Diameter) Maximum

Height 						: 320 mm Maximum

| SR NO. |  | TECHNICAL SPECIFICATION | MAKE |  |
| --- | --- | --- | --- | --- |
| 1 |  | Main Motor (1.5 HP) Capper Motor (0.25 HP) Cap Feeder (0.25 HP) Conveyor Motor (2.0 HP) | ROTOMOTIVE |  |
| 2 |  | Switch Gear | SCHNEIDER |  |
| 3 |  | PLC Relay Card 230 V AC | DELTA |  |
| 4 |  | Material of Construction | SS 304 |  |
| 5. |  | Bearing | SKF |  |
| TECHNICAL DETAILS |  |  |  |  |
| 1 | Main Drive Motor |  |  | 5 HP (3.0 KW) x 1440RPM x 415 VAC. |
| 2 | Cap Hopper Gerard Motor |  |  | 0.25 HP (0.33 KW) x 1440 RPM x 415 VAC Hopper Capacity 1000 Caps At a time Material of contraction SS304 SS 304 Quality belt ¾” Size Size (L x W x H ) - 3 feet 3 feet x 7.5 Feet Weight = 200 kg Approx. |
| 3. | Electrical Control Panel |  |  | Inputs - Photo Sensor / Limit switch Output - 230 VAC Valves |
| 4. | Filling Valves |  |  | ½” BSP 230 VDC |
| 5. | Proximity Switches |  |  | 24 VDC M18 Size (For No Bottle No Fill System) |
| 6. | SS Shrink Tunnel |  |  | 4KW (440 VAC) |
| 7. | Conveyor 16 feet (Air Conveyor) and 24 feet (Slat Conveyor) Geared Motor |  |  | 0.25 HP 32 RPM 440 VA Material of construction SS304 Belt = ¾” Size material SS304 Adjustable legs for different size of bottle up to 300mm |
| MACHINE DIMENSONS |  |  |  |  |
| 1. | Only Machine (L x W x H) With Conveyor &All Accessories (L x W x H) |  |  | 6’ x 7’ x 6’ (FEET) 20’ x 11’ x 6’ (FEET) |
| WOODEN CARATE PACKING DIMENSIONS |  |  |  |  |
| 2. | Main Machine (L x W x H) Net Weight Gross Weight |  |  | 98” x 98” x 102” (INCHES) 1800 KG2700 KG |
| 3 | Accessories (L x W x H) Net Weight Gross Weight |  |  | 140” x 34” x 42” (INCHES) 800 KG1000 KG |

SPECIAL FEATURE OF MACHINE

- Bottle Catching & All Star Wheels below the Neck Only.

- Instruction & Indication Systems.

- All Machine Part With Cnc Made.

- Programmable Logic Control With Hmi.

- Filling Water Tank Low & High Level Inter Lock.

- In-feed Air Blower Conveyor Ss 304.

- All Material Ss 304

- Cap Hooper Elevator Type & Having Capacity Of Min. 1000 Caps At A Time

- Out-feed Conveyor Ss 304.

| Automatic Sticker Labeling Machine for flat bottle |  |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| The Automatic Vertical Sticker Labeling Machine is one of the most users friendly. The machine is used for full/partial labeling on different size of flat or rectangular containers. The automatic model is equipped with roller spacing device resulting in a simple operation with “No change parts required”. |  |  |  |
| Operation |  |  |  |
| The filled and sealed bottle is conveyed from the Conveyor and on the same conveyor one bottle separator attachment is provide to separate the bottle from continuous movement. A sensor provided to sense the product after sensing the sensor give command to operate steeper or servo motor to release label when one label is stick the gap sensor give command to stop the servo or stepper during releasing of label provision of brush to press the label & remove the air between label & bottle after completion of the cycle one operation of labeling is finish & conveyor bring the bottle for next operation.. |  |  |  |
| Technical Specifications |  |  |  |
| Output/Hour* |  |  | 4000 to 6000 |
| Direction of Movement |  |  | Left to Right |
| Controlling |  |  | Microprocessor base control |
| Power Consumption | Conveyor motor |  | 0.5 HP / 415 V / 50 HZ |
|  | Pressing device motor |  | 0.25 HP / 415 V / 50 HZ |
|  | Stepper motor |  | 60 kg / cm2 |
| Height of Conveyor |  |  | 850 mm to 900 mm Adjustable |
| Machine Dimensions** |  |  | 2100 mm (L) x 900 mm (W) x 1200 (H) Approx. |
| Case Dimensions |  |  | 2200 mm (L) x 1000 mm (W) x 1325 (H) Approx. |
| Net Weight |  |  | 350 Kgs. Approx. |
| Gross Weight |  |  | 600 kgs Approx. |
| Input Specification |  |  |  |
| Label Specifications |  | Length** | 240mm Max |
|  |  | Roll Diameter | Max 300 mm & ID 75 mm |
|  |  | Space between two label | Min. 3 mm anti clock wise |
| * Depends on Container size, fill size, Neck diameter of container, and nature of liquid etc. ** We can supply as per customer requirements. |  |  |  |
| Salient Features |  |  |  |
| - High Production Speeds. - SS steel structure cladded with stainless steel 304 sheet to meet GMP norms. - Microprocessor Based operation - Accurate label placement. - No change parts requirements. - Very less down time for change over. |  |  |  |

| DIGITAL BATCH CODING MACHINE |  |
| --- | --- |
|  | - Stainless Steel Body - The Machine Can Be Operational Under The Temperature Conditions Between 0 To 40c - Print head Is Closer To The Substrate - Inverter Mechanism Which Automatically Reciprocate The Ink Even During Non- Operational Time. - Printing Mode-Continuous ( Change Control System) - Character height-1 to 10 mm - Print Speed - Max. 2666 characters/sec. - Ink Type - Food grade ink - Character Type - Alphabet, figures, symbols |

SEMI AUTO SHRINK WRAPPING MACHINE

Web Sealer Specification

| Sealing Dimension | 24" Sealing Arm For Sealers |
| --- | --- |
| Power Consumption | 18 KW |
| Machine Output | 5 Pack/Minute - Matrix size 4 X 3 |
| Infeed Conveyor | SS 304 Plated Sheet |
| Sealing Knife | Continuously heating Teflon Coated sealing Knife 24" film guard (thermostatically control) |
| Type Of Sealing | Web Sealing 2 Rolls with The Side Open. |
| Pneumatic | Janatics/Pneumax |
| Machine Construction | 16 Gauge M.S. Body Fabrication with Powder Coated Color. All Contact Parts are SS304 |
| Film Carriage Roller | Fully Stainless Steel Material with Buffing |
| Air Consumption | Constant Air Pressure 100 PSI, 15CFM @7-8KG / Air req. |
| Tunnel Conveyor | SS Mesh Chain Conveyor |
| Motor & Gear Box | 1 HP Ac motor with gear box |
| Tunnel Opening In Mm | 1550 L X 600 W X 400 H |
| Heating Chamber | C-Chamber Heating Zone - 1550mm Length Insulation of Glass Wool thickness Of 65 mm, Heat not generate of outer Body for continuous running of Machine. |
| Tunnel Conveyor Speed | variable speed |
| Heater Specification | 700mm Long, 1.5 KW SS Tube Fined Heater (12 nos. Heater) |
| Total Dimension In Mm | 4250 L X 1250 W X 1750 H ( Web Sealer shrink tunnel) |
| AC Motor | 1 HP 2800 RPM motor with 7" Impeller |
| Blower Fan | 0.25 HP AC motor 2800 RPM 2 no. with Aluminum Blower Fan for long performance |
| Cooling Zone | In exit for Tunnel provide Cooling Fan for finishing the final product. |
| Idler Conveyor | For Idling the product. (Drop down). Roller Conveyor 1000mm L |

| Price Breakup |  |  |  |  |
| --- | --- | --- | --- | --- |
| S.N. | PARTICULERS | CAPACITY | QTY | AMOUNT |
| 1 | Fully Automatic Bottle Rinsing, Filling, Capping Machine + Inspection Screen | 30 BPM | 1 | 8,50,000/- |
| 2 | Automatic Flat bottle Sticker Labeling Machine | 100 bpm | 1 | 2,80,000/- |
| 3 | Bach Coding Machine (Ink Jet Printer Videojet make) | 1 SET | 1 | 2,25,000 |
| 4 | Shrink Wrapping Machine | 1 SET | 1 | 2,85,000 |
|  | BANK DETAIL : Account name :- TORQ PACKAGING SOLUTION Account number :- 8000032128 Bank name :- KOTAK MAHINDRA BANK LTD Branch :- KATHWADA IFSC :- KKBK0002606 COMPANY GSTIN : 24ARDPD2565K1ZX | TOTAL |  | 16,40,000/- |
|  |  | GST @ 18% |  | 2,95,200/- |
| TOTAL AMOUNT |  |  |  | 19,35,200/- |
| TERMS & CONDITIONS |  | For, TORQ Packaging Solution |  |  |
| - 50 % Advance on Signing up of Contract - 50% On Completion of Machinery Manufacturing Before Dispatch / During Inspection of machinery - Order Once Placed Cannot be Cancelled - No Refund will be entertained |  |  |  |  |